# Debug ESP32S3 using PowerShell

# Target hardware

- ESP32-S3 Super Mini Black Gold

- Clone of a ZERO board

- Has 18 pins, boot and reset buttons and a USB-C connector

- MCU is ESP32-S3 FH4R2

- Datasheet https://www.espressif.com/sites/default/files/documentation/esp32-s3_datasheet_en.pdf

- CPU is Xtensa® dual-core 32-bit LX7 microprocessor

- Clock speed: up to 240 MHz

- Flash is 4Mb, PSRAM 2Mb and SRAM 512 Kb

- Assume USB JTAG/debug unit (interface 0) set as COM3

- Assume USB JTAG/debug unit (interface 2) configured as a Universal Serial Bus Device called USB JTAG/Serial debug unit

# Configuring the USB ports

- Use Zadig to configure USB JTAG/debug unit (interface 0) as usbser

- Use Zadig to configure USB JTAG/debug unit (interface 2) as WinUSB

# Install ESP-IDF

- Download the offline tools installer from https://dl.espressif.com/dl/esp-idf/?idf=4.4

  - Either a specific version, or use the universal online installer and choose the version

  - Idf-env.exe runs so OK it

- During the installation two special shortcuts (CMD & PowerShell) will be added to the desktop

  - Both shortcuts setup the Python environment ready for ESP-IDF commands

  - I prefer PowerShell as can cd to a new path in one step

- Updating ESP-IDF tools and adding ESP-IDF tools to PATH using an export script

  - Open the PowerShell shortcut and run install.ps1 and then export.ps1

  - Open the Command shortcut and run install.bat and then export.bat

  - Look at the output and run any other suggestions for missing items

# Building the code

- Copy the blink example from E:\Users\Steven\Downloads\esp-idf-v5.2.2\esp-idf-v5.2.2\examples\get-started\blink to E:\Users\Steven\Documents\GitHub\ESP-IDF\blink

- Open a special ESP-IDF 5.2 PowerShell window, do not use the CMD version as less flexible.

- Change to blink directory using cd E:\Users\Steven\Documents\GitHub\ESP-IDF\blink

- Set the target idf.py set-target esp32s3

- Open Menuconfig idf.py menuconfig

- Enter Serial flasher config

- Change flash size from 2MB to 4MB

- Change any other settings as needed

- Build the code using idf.py build

# Uploading the code

- Upload the built binaries to the ESP32S3 idf.py -p COM3 flash

- JTAG upload can be used as an alternative to serial via a COM port using either of the following:

  - openocd -s scripts -f interface/esp_usb_jtag.cfg -f  board/esp32s3-builtin.cfg -c "program_esp E:/Users/Steven/Documents/GitHub/ESP-IDF/blink/build/blink.bin 0x10000 verify reset exit"

  - openocd -s scripts -f board/esp32s3-builtin.cfg -c "program_esp E:/Users/Steven/Documents/GitHub/ESP-IDF/blink/build/blink.bin 0x10000 verify reset exit"

# Starting OpenOCD the Open On-Chip Debugger server

- Create a file called gdbinit in the blink folder at E:\Users\Steven\Documents\GitHub\ESP-IDF\blink

- Edit gdbinit to include the following text:

target remote :3333

set remote hardware-watchpoint-limit 2

mon reset halt

maintenance flush register-cache

thb app_main

c

- Open a special ESP-IDF 5.2 PowerShell window, do not use the CMD version as less flexible.

- Check OpenOCD working using openocd --version

- Change to blink directory using cd E:\Users\Steven\Documents\GitHub\ESP-IDF\blink

- Start OpenOCD using openocd -f board/esp32s3-builtin.cfg

- OpenOCD Responds with:

Open On-Chip Debugger v0.12.0-esp32-20240318 (2024-03-18-18:26)

Licensed under GNU GPL v2

For bug reports, read

http://openocd.org/doc/doxygen/bugs.html

Info : only one transport option; autoselecting 'jtag'

Info : esp_usb_jtag: VID set to 0x303a and PID to 0x1001

Info : esp_usb_jtag: capabilities descriptor set to 0x2000

Info: Listening on port 6666 for tcl connections

Info : Listening on port 4444 for telnet connections

Info : esp_usb_jtag: serial (24:58:7C:DC:25:C0)

Info : esp_usb_jtag: Device found. Base speed 40000KHz, div range 1 to 255

Info : clock speed 40000 kHz

Info : JTAG tap: esp32s3.cpu0 tap/device found: 0x120034e5 (mfg: 0x272 (Tensilica), part: 0x2003, ver: 0x1)

Info : JTAG tap: esp32s3.cpu1 tap/device found: 0x120034e5 (mfg: 0x272 (Tensilica), part: 0x2003, ver: 0x1)

Info : [esp32s3.cpu0] Examination succeed

Info : [esp32s3.cpu1] Examination succeed

Info : starting gdb server for esp32s3.cpu0 on 3333

Info : Listening on port 3333 for gdb connections

Info : [esp32s3.cpu0] Debug controller was reset.

Info : [esp32s3.cpu0] Core was reset.

Info : [esp32s3.cpu1] Debug controller was reset.

Info : [esp32s3.cpu1] Core was reset.

Info : [esp32s3.cpu0] Target halted, PC=0x40379532, debug_reason=00000000

Info : [esp32s3.cpu0] Reset cause (1) - (Power on reset)

Info : [esp32s3.cpu1] Target halted, PC=0x40379532, debug_reason=00000000

Info : [esp32s3.cpu1] Reset cause (1) - (Power on reset)

# Starting GDB the GNU Debugger

- Open a second special ESP-IDF 5.2 PowerShell window

- Change to blink directory using cd E:\Users\Steven\Documents\GitHub\ESP-IDF\blink

- Start GDB using xtensa-esp32s3-elf-gdb.exe build/blink.elf -x gdbinit

# Using GDB commands

- For a variety of different layouts within the shell run layout next

  - To step through assembly code use nexti or stepi

  - To step through lines of code use next or n or step or s

  - With the code layout the stepping or nexting can be seen line by line

- Stepping one line of code, including stepping into subroutine s or s1

- Stepping multiple lines of code, including stepping into subroutine s7

- Set a breakpoint break blink_example_main.c:106 or just break 106

- To list current breakpoints and times hit info break

- To delete a breakpoint del 3 where 3 is the breakpoint, not line, number

- To delete a breakpoint dis 3 where 3 is the breakpoint, not line, number

- To enable a breakpoint en 3 where 3 is the breakpoint, not line, number

- Continue execution to the next breakpoint c or continue

- To step to the next line of code, and stepover subroutines n or next

- To view the source code lines around the current line list or l

- To examine a variable print s_led_state or print count

- To set the value of a variable set s_led_state=1 or count=146

- To exit GDB, use the quit command q

# Viewing serial output

- The blink example prints Turning the LED ON! and Turning the LED OFF!

- To view open a serial terminal e.g. SmarTTY and connect to COM3 at 115200 baud

# Stopping GDB the GNU Debugger

- To exit GDB, use the quit command q

# Stopping OpenOCD the Open On-Chip Debugger server

- To shut down the OpenOCD server Ctrl C

# For more info about using GDB see:

- https://docs.espressif.com/projects/esp-idf/en/v5.2.2/esp32/api-guides/jtag-debugging/index.html

- https://docs.espressif.com/projects/esp-idf/en/v5.2.2/esp32/api-guides/jtag-debugging/debugging-examples.html

- https://ftp.gnu.org/old-gnu/Manuals/gdb/html_node/gdb_toc.html

- https://web.eecs.umich.edu/~sugih/pointers/gdbQS.html

- https://www.youtube.com/playlist?list=PL9IEJIKnBJjHGWPN_S9NS_Ky1-tC8ZrUI